Propagation Delay: Bis das ganze Auto raus ist > first bit => size / bandwith

Transmission delay: Wie lange die ganze reise dauert für 1 bit => distance / speed

Gesamt: Prop. + Trans.

Processing delay: processing of router / host / switch (const var)

Queuing delay: How long stuff waits in queue before sent

| Bereich | Präfix | Bedeutung | Geroutet? |
| --- | --- | --- | --- |
| Global Unicast | 2000::/3 | Öffentliche IPv6-Adresse | Ja |
| Unique Local (ULA) | fc00::/7 (meist fd..) | Privat | Nein |
| Link-Local | fe80::/10 | Nur lokales Netz | Nein |
| Multicast | ff00::/8 | Multicast | Nein |
| Loopback | ::1 | Loopback | Nein |

RTT =  t_to_reciever + t_to_sender (t * 2 most of the time)

| Layer | Name | Example Technology / Protocol | PDU name | One Function of the Layer |
| --- | --- | --- | --- | --- |
| 7 | Application | HTTP, FTP, SMTP, DNS | Data | Provides network services directly to user applications |
| 6 | Presentation | TLS/SSL, JPEG, MPEG | Data | Data representation (encryption, compression, encoding) |
| 5 | Session | NetBIOS Session Service, RPC | Data | Session establishment, management, and termination |
| 4 | Transport | TCP, UDP | Segment (TCP) / Datagram (UDP) | End-to-end communication, flow & error control |
| 3 | Network | IP, ICMP, IPsec | Packet | Logical addressing and routing between networks |
| 2 | Data Link | Ethernet (IEEE 802.3), Wi-Fi, ARP | Frame | Framing, MAC addressing, error detection |
| 1 | Physical | Ethernet PHY, RS-232, fiber optics | Bit | Transmission of raw bits over the physical medium |

| Msg | ACK | SYN | FIN | Payload len | Seq | Ack |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 3247 | 0 |
| 2 | 1 | 1 | 0 | 0 | 7 | 3248 |
| 3 | 1 | 0 | 0 | 0 | 3248 | 8 |
| Msg | ACK | SYN | FIN | Payload len | Seq | Ack |
| 4 | 0 | 0 | 0 | 130 | 50 | 500 |
| 5 | 1 | 0 | 0 | 200 | 500 | 180 |
| 6 | 1 | 0 | 0 | 130 | 180 | 700 |
| 7 | 1 | 0 | 0 | 20 | 700 | 310 |
| Msg | ACK | SYN | FIN | Payload len | Seq | Ack |
| 8 | 0 | 0 | 1 | 0 | 1 | 2342 |
| 9 | 1 | 0 | 0 | 0 | 2342 | 2 |
| 10 | 0 | 0 | 1 | 0 | 2342 | 2 |
| 11 | 1 | 0 | 0 | 0 | 2 | 2343 |

| Bereich | Erkennung |
| --- | --- |
| Multicast | 224–239.x.x.x |
| Broadcast | 255.255.255.255 |
| Loopback | 127.x.x.x |
| Link-Local | 169.254.x.x |
| Private | 10.x.x.x / 172.16–31.x.x / 192.168.x.x |

Baud: signals per second

NRZ: ___|-----|___ <- geht nicht wieder runter bei mehreren gleich; Line Code: _ = 0, - = 1 (das mapping selber, NRZ ist ein line code)

| Message 1: 10.0.0.4:444555 | 193.104.220.7:433 |
| --- | --- |
| Message 2: 194.95.83.45:56478 | 193.104.220.7:443 |
| Message 3: 193.104.220.7:443 | 194.95.83.45:56478 |
| Message 4: 193.104.220.7:443 | 10.0.0.4:444555 |

8-Bit payload => 7 Bits are beeing appended. (app = len(payload) – 1)

Message: 11010011 0000000; Errors detect: d – 1; Errors correct: floor((d-1) / 2)

POTS frequence: 3100Hz; SNR = P_signal / P_noise

C = B * log2(1 + SNR); C: bit/s, B: Hz, SNR: -

Line Coding ist für Basisband-Übertragung gedacht, aber die Telefonleitung ist ein Bandpasskanal (ca. 300–3400 Hz), daher müssen die Daten per Modulation auf eine Trägerfrequenz gelegt werden, um übertragen werden zu können.

Bit Stuffing: When HDLC discovers 5x 1 bits in the payload / body, it puts a 0 at the end of the 5x 1s. This is undone upone recieving.

```

110100110000000 ^

100101

100011

100101

110100

100101

100010

NRZ has the problem that no signal change can bring the listener / reciever out of tact / rythm when transmitting the same type of bits back to back (Baseline Wanderer). Manchester solves this. The process of solving this problem without another tick channel is called clock recovery. The process of mapping electrical signals to bits is called Modulation. Indicating the bits not by voltage but by change of voltage (like manchester) is called Alternative Mark Inversion (AMI).

100101

111000

100101

111010

100101

------

011111

```

The ARP - Adress Resolution Protocol works by sending a broadcast containing the IPv4 Address of the target device. Each device compares this to its own IPv4 Address; If it matches, the device sends an ARP response via unicast containing its MAC address back to the sender. ARP only has one job: Mapping a MAC address to a IPv4 Address. NDP does alot more. ARP uses Broadcast instead of multicast like NDP, causing extreme security riscs and more network load. NDP uses SLAAC to determine an IPv6 address for you. NDP can find routers on its own. ARP is used for IPv4 and more, NDP is specifically designed for IPv6. (ARP was also designed for IPv4, but can be used for a lot of other things, unlike NDP)

Ethernet cable types

| Method | Used in | How it works | Needs server? |
| --- | --- | --- | --- |
| Address Autoconfiguration | IPv4/IPv6 | Auto IP, general term | maybe |
| DHCP | IPv4/IPv6 | Ask server → get IP + info | Yes |
| SLAAC | IPv6 | Listen Router Advertisement → make own IP | No |

twisted pair cables (e.g. Cat5/Cat6), fiber-optic cables, and historically coaxial cables.

Difference between coaxial and twisted pair cables

Coaxial cables use a single shielded conductor, while twisted pair cables use two twisted wires to reduce electromagnetic interference.

Advantages of fiber-optic cables over copper

Fiber-optic cables provide higher bandwidth, longer transmission distances, and are immune to electromagnetic interference.

Transmission range vs. interference range

transmission range is the area where a signal can be correctly received; Interference range is where the signal can still disturb other comms.

Wireless network challenges

Wireless networks suffer from interference caused by other transmitters and signal attenuation due to distance and obstacles.

Bridges simply connect 2 or a few more hosts directly together; Switches connect multiple, are faster and full duplex (bi-directional). Forwarding tables save the last send messages from host to host. If no entry for a host exists, the bridge floods to all connected ports except the reciever. Learning bridges forward data dynamically by observing source MAC addresses. Bridges use the Spanning Tree Protocol STP to avoid loops. Root Bridge has the lowest Bridge ID and highest priority.

Media Access Controle (MAC) is used to coordinate multiple devices without collision. The MAC number is burned into the hardware (48 bit global std; first 24bit manufac, other Seriesnum). Both ethernet and wlan are non-determ., ether uses CSMA/CD, wlan uses CSMA/CA. /CD tries to detect collisions, CA tries to avoid them (like TCP slowstart). In CA additional RTS / CTS frames are used to reserve the medium and avoid collisions. A higher p value is used for low loads and higher more aggressive transmission with less delay.

An autonomous system (AS): collection of IP networks under a single administrative control that share a common routing policy.

Distance-vector: Bellman-Ford, knows neighbours and distance to every target.

Link-state: Dijkstra, knows the whole network. (Both are Routing Protocol classes)

BGP (Distance vec) is a protocol for Inter-AS routing, pathing is based on policy and neighbour AS‘s.

OSPF (link-state) is a protocol for Intra-AS routing, knows the whole network.

RIP (Distance vec) is a protocol for Intra-AS routing, only knows neighbour, paths based on hops to target.

In practice, both BGP and OSPF are used together.

Why omnidirectional transmission is still used

Omnidirectional transmission is cheaper, simpler, and allows mobile devices to connect without precise antenna alignment.

Why attenuation affects wireless networks more

Wireless signals spread freely through space and are weakened by obstacles, while wired signals are guided and shielded inside cables.

Popular network technologies in companies or universities

Ethernet (LAN) and Wireless LAN (WLAN).

Communication model used by eduroam

Eduroam uses a client–server communication model with centralized authentication.

Frequency spectrum used by WLAN

WLAN operates mainly in the 2.4 GHz and 5 GHz ISM bands, and also in the 6 GHz band with Wi-Fi 6E.

Higher transmission power: Bluetooth or WLAN

WLAN achieves higher trans. power because it´s designed for high data rates and long range; Bluetooth is optimized for low energy.

IEEE 802.11b-WLAN has a data rate of 11Mbit/s

| Protocol | Type | How it works | Used in |
| --- | --- | --- | --- |
| ALOHA | Random | Send anytime, if collision → try again | Early wireless, satellite |
| CSMA/CA | Random | Check channel, wait if busy, use RTS/CTS | Wi-Fi |
| MACA | Random | Use RTS/CTS to avoid hidden node collision | Wireless LAN |
| TDMA | Time | Each user has own time slot | GSM, cellular, satellite |
| FDMA | Frequency | Each user has own frequency band | Radio, old mobile |
| CDMA | Code | All use same frequency, each has unique code | 3G mobile, GPS |

Subnet ID = Network IP => BITWISE_AND(Mask, IP)

Package stays in network if (NetworkIP_reciever == NetworkIP_sender)

MTU: Maximum Transmission Unit; max size of package, if the size-to-be-send exceeds this, its (IP-) fragmented, meaning that

every package gets its own header. => more overhead, if one fragment is lost, the whole package needs to be sent again.

| Type | Range / Purpose |
| --- | --- |
| PAN | Personal Area Network, e.g., Bluetooth, few meters |
| BAN | Body Area Network, e.g., medical sensors on the body |
| LAN | Local Area Network, e.g., office or building, high speed |
| MAN | Metropolitan Area Network, e.g., city-wide network |
| WAN | Wide Area Network, e.g., Internet, connects cities/countries |

| Type | Description |
| --- | --- |
| Unicast | 1-to-1 → sent to a single recipient |
| Broadcast | 1-to-all → sent to all devices in the network |
| Multicast | 1-to-group → sent only to a specific group of recipients |
| Anycast | 1-to-nearest → sent to the nearest/optimal node in a group |

Synchronous: sender and receiver are time-synchronized, data sent at fixed intervals. Asynchronous is not fixed.

| Mode | Description |
| --- | --- |
| Simplex | one-way only → sender → receiver |
| Half-Duplex | both ways, but alternating |
| Full-Duplex | both ways simultaneously |

ICMP: Internet Control Message Protocol; used when errors occur in IP-Networks (e.g. TTL (Time to live) ran out, package not reachable). Pings

a ICMP Echo Request to the target => the target awnsers with ICMP Echo Reply; This tests if the host is reachable and how long the response takes.

Traceroute: uses incrementing TTL to discover route to target, resets when TTL reaches 0, starting the loop again.

| Algorithm | Order of the nodes being read |
| --- | --- |
| Bellman-Ford | No order, iterates over all nodes till there are no changes in a full iteration. |
| Dijkstra | Node with the smalles distance to start-point is viewed next, uses priority queue. |

TCP (with AIMD), UDP & QUIC:

Silly Window Syndrome occurs when small segments of data are sent inefficiently, causing excessive overhead and reduced network performance.

Avoidance techniques prevent sending very small segments until enough data is accumulated or the receiver’s window is sufficiently large.

Two Windows are contained when using TCP, one for the network capacity and one for the capacity of the reciever.

Fast Recovery is optional and activated by the reciever by sending 3 ACKs back-to-back, causing the threshhold of the slow start to halve.

Threshholds start at big sizes, e.g. 64kB. Upon timeout, the cwnd (current window size) is dropped back to 1 (not zero!). When the threshhold is

reached without timeout, TCP switches to the congestion-avoidance-phase / linear mode, setting MSS = 1kB. TCP does not stop „exploring“ the

bounderies of the reciever.

TCP is Connection-Oriented (First Connection, then send data).

Congestion can occur upon exceeding the network capacity or by overloading routers / switches.

t_congestion = RTT *  rounded_up( log2( size_window / size_segment ) )

UDP just sends stuff and forgets. (Connectionless)

Protocols which use UDP, like DNS, switch back to TCP when UDP isnt enough anymore (e.g. segment too big).

UDP is used when speed mathers more then security e.g. livestreams & videogames.

QUIC is a more secure protocol based on UDP. Developed by Google. Uses multiplexing and implements Head-of-Line-blocking  and HTTPS (like TCP).

DNS maps Domain to IP, which is stored in the Resource Record (RR). Each part of a domain („www“, „google“, „.com“, etc.) is called a Label.

The Zone is a subset of the DNS-Namespace, which controls other RRs of this domain. This is the index (google.com / youtube.com) most of the time.

FQDN: Full qualified Domain name, e.g. www.google.com

SYN Flood: Send giant amount of Syn requests to a host, causing it to open a lot of windows, blocking them for other users (Denial of Service / DOS)

The hidden terminal problem is a wireless networking collision issue where two nodes (A and C) outside each other’s range both transmit to a commonreceiver (B), causing signal collision at B. Because A and C cannot hear each other, they incorrectly sense the channel as free.